# Fairydust Status Notes

## Current Fairydust Status

- Fairydust is the experimental Asahi Linux kernel branch for USB-C DisplayPort Alt Mode support on Apple Silicon.

- Basic USB-C external display output works in some configurations.

- Fairydust is still considered work in progress and is not production-ready.

- Current development focus is reliability, compatibility, and feature parity rather than first-time display output.

- Main remaining areas:

  - USB-C / DP Alt Mode negotiation

  - hot-plug and reconnect reliability

  - link training

  - monitor and adapter compatibility

  - suspend/resume

  - display routing

  - DCP firmware handling

  - USB4 / Thunderbolt

  - PCIe tunneling

- Latest Fairydust branch update: September 8, 2026.

- Latest branch changes included an M1 SIO Device Tree fix that enabled DisplayPort audio functionality.

## Issue #634 - USB-C DP Hot-Plug Failure

### Status

- Open issue.

- Affects USB-C DisplayPort reconnect behavior.

- Reported on M1 Pro hardware.

### Behavior

- External display works initially.

- After unplugging and reconnecting, the display may remain black until reboot.

- Two separate failure causes identified.

### Cause 1: DCP False Crash Detection

- iomfb_poweroff() waits 50 ms for a display shutdown clear-swap operation.

- Hardware was observed taking approximately 53 ms.

- Driver interprets the timeout as a DCP crash.

- dcp->crashed becomes permanently set.

- Future display commits return -EINVAL.

- Monitor can still be detected, but no mode is applied.

- Reboot required to recover.

### Related PRs

- PR #582:

  - still open

  - avoids permanently setting DCP as crashed after this timeout

  - successfully tested on M1 hardware

- PR #635:

  - addressed the same general timeout behavior

  - closed without merge

### Cause 2: CD321x / USB-C Controller State Failure

- USB-C controller can fail to enter DisplayPort Alt Mode again after certain USB failures.

- Power and USB may still partially function.

- No proper DisplayPort negotiation occurs.

- No DP connection or HPD sequence.

- Still under investigation.

### Relevance

- Directly related to:

  - USB-C handshake

  - DP Alt Mode negotiation

  - reconnect reliability

  - original project goals 1 and 2

## Issue #611 - DisplayPort Link Training / Signal Strength

### Status

- Open issue.

- Fairydust detects and configures the monitor, but actual display output can remain black.

### Working Parts

- monitor detected

- EDID read successfully

- display modes enumerated

- DisplayPort link configured

- DCP modeset completes

- DRM reports connector as connected

### Failure

- monitor remains black despite successful software-side setup

### Suspected Cause

- DisplayPort sink requests specific:

  - voltage swing

  - pre-emphasis

- Fairydust stores the requested values.

- Requested values are not fully applied to the ATC PHY hardware.

- Physical link signal may therefore not meet the monitor’s requirements.

### Observed Effects

- one monitor may work while another does not

- one cable may work while another does not

- DP 1.2 may work while DP 1.4 fails

- lower resolutions may be more stable than higher-bandwidth modes

### Relevance

- Strong fit with:

  - stable display output

  - higher resolution / refresh support

  - feature parity

  - original project goals 2 and 3

## Issue #571 - Adapter / Hub Link Bounce

### Status

- Open issue.

- Reported on M2 MacBook Air.

### Behavior

- Same machine, monitor, and port behave differently depending on adapter.

- UGREEN adapter:

  - stable

  - external display works

- VIA Labs-based hub:

  - display detected

  - EDID read

  - modes found

  - link repeatedly disconnects and reconnects

### Failure Pattern

- connected

- HPD asserted

- display modes found

- link drops

- reconnect begins

- cycle repeats

### Relevance

- Shows incomplete interoperability.

- Useful for testing:

  - adapters

  - hubs

  - docks

  - DP-to-HDMI conversion

  - reconnect stability

- Directly relevant to feature parity.

## Issue #579 - Apple Studio Display Failure

### Status

- Open issue.

### Behavior

- Fairydust brings up most of the USB-C / DisplayPort path.

- DCP firmware sends:

  - DPTX_APCALL_INACTIVE_SINK_DETECTED

  - call 20

- Fairydust does not have proper handling for this event.

- Driver acknowledges the event without performing the correct response.

- DCP then tears down the display connection.

### Possible Cause

- Studio Display likely requires a sink activation or wake-up sequence.

- macOS behavior for this condition is not yet fully reproduced in Linux.

### Possible Investigation Method

- m1n1 tracing

- compare macOS hardware/firmware behavior with Fairydust

### Relevance

- DCP firmware reverse engineering

- display initialization

- advanced monitor compatibility

## Issue #474 - Monitor Identification / Layout Not Updating

### Status

- Open issue.

### Behavior

- Switching between external monitors may retain stale display information.

- Screen make/model may not update correctly.

- Desktop layout may stay based on previous monitor dimensions.

- KDE arrangement can show overlapping screens or gaps.

### Relevance

- Lower priority than kernel-level link problems.

- Still part of user-visible feature parity.

## Issue #637 - Multi-Port / Blessed Port Routing

### Status

- Opened September 22, 2026.

- Closed September 26, 2026.

- Closed as “not planned” because Asahi developers stated they intend to implement the correct solution themselves.

### Original Limitation

- Fairydust statically routes DisplayPort to one specific “blessed” USB-C port.

- Other USB-C ports are not dynamically assigned for display output.

### Experimental Progress

- Contributor successfully changed DCP routing between USB-C ports.

- Display output worked on multiple ports.

- Hot-plug and cold-plug were reported working in tests.

### Remaining Architectural Work

- proper per-port DRM connectors

- dynamic DCP/DCPEXT assignment

- crossbar routing

- power-domain management

- correct Device Tree bindings

- HDMI coexistence

### Relevance

- Important current Fairydust limitation.

- Less suitable as a primary team target because upstream Asahi developers are actively planning their own implementation.

- Useful as a future Mac mini validation target.

## Suspend / Resume Problems

### Current State

- DisplayPort suspend/resume handling is incomplete.

- Some ATC PHY power domains are forced to remain powered.

### Current Workaround

- Device Tree uses apple,always-on for certain display-related hardware.

- Prevents full shutdown of some hardware during suspend.

### Desired Behavior

- display active

- system suspends

- hardware powers down correctly

- system resumes

- PHY and display path reinitialize correctly

- external monitor reconnects automatically

### Related PR

- PR #510:

  - handles inactive DCPs during suspend

  - attempts to prevent resume crashes

  - still requires more stress testing

### Relevance

- Feature parity

- power management

- display reliability

## USB4 / Thunderbolt Status

### Recent Development

- Sven Peter submitted version 2 of an Apple Silicon USB4/Thunderbolt driver series in September 2026.

- Series contains 22 patches.

- Targets M1, M2, and M3 SoCs.

### Main Hardware

- Apple ACIO block

- Apple USB4 router

- internal Cortex-M3 coprocessor

- DART/IOMMU

- Native Host Interface

### Current Functionality

- USB4 infrastructure brought up

- Linux Thunderbolt software connection manager can interact with Apple hardware

- initial XDomain support

- USB3 tunneling over USB4

### Still Incomplete

- full DisplayPort tunneling

- PCIe tunneling

- complete Thunderbolt feature parity

- complete hot-plug / device compatibility

- performance optimization

### Relevance

- Direct connection to later project goals.

- Reduces amount of USB4 foundation that would need to be implemented from scratch.

## PCIe Tunneling

### Target Path

Thunderbolt NVMe enclosure

→ USB4 host/router

→ Thunderbolt connection

→ PCIe tunnel

→ external PCIe NVMe controller

→ Linux PCI enumeration

→ NVMe driver

→ storage device available

### Validation Possibilities

- device appears in lspci

- NVMe driver binds

- drive appears in Linux

- read/write tests

- hot-plug tests

- throughput tests

- stability tests

### Status

- Not fully completed in current Apple Silicon USB4 support.

- Remains a valid stretch goal.

## Strong Remaining Areas for Project Work

- USB-C DP negotiation failures

- hot-plug and reconnect reliability

- CD321x controller recovery

- DisplayPort link training

- voltage swing / pre-emphasis handling

- monitor compatibility

- adapter / dock compatibility

- DCP firmware event handling

- suspend/resume

- Mac mini-specific display behavior

- validation of future multi-port routing

- USB4 integration

- PCIe tunneling

- performance and reliability testing

### Possible Approach

- reliable USB-C / DP Alt Mode negotiation

- stable external display output

- proper link training and interoperability

- reliable higher-bandwidth display modes

- USB4 / Thunderbolt bring-up

- PCIe tunneling

- throughput and reliability optimization